Safety Razor & Shaving Brush Holder

CAD Customization, OpenSCAD Parametric Design & Slicing Guide

Personalized for Tumesh | Multi-Brush OpenSCAD System

# 1. Executive Summary & Overview

This project delivers a complete, personalized 3D printing system for the all-in-one 'Safety Razor Holder + Soap, Brush and Blades' bathroom organizer. It includes pre-rendered production models for multiple knot diameters (24 mm, 28.5 mm, 30 mm) as well as an interactive OpenSCAD Customizer (.scad) design that lets you generate tailored holders for your entire shaving brush collection on demand.

# 2. Pre-Configured Model Specifications

| Variant | Knot Opening | Slot Width | Arm Thickness | Recommended Brush Knot |
| --- | --- | --- | --- | --- |
| 30.0 mm | 30.0 mm | 30.0 mm | 16.0 mm | 28 mm - 29 mm knots (spacious fit) |
| 28.5 mm | 28.5 mm | 28.5 mm | 16.75 mm | 26 mm - 27 mm knots (medium fit) |
| 24.0 mm | 24.0 mm | 24.0 mm | 19.0 mm | 22 mm standard knots (+ 2 mm play) |

# 3. OpenSCAD Interactive Customizer Design

To easily generate custom holders for different shaving brushes in your collection, the project includes 'customize_holder.scad'. This script integrates directly with OpenSCAD's built-in graphical Customizer panel, enabling you to adjust dimensions, labels, and emblems without writing code.

## 3.1 Key Parametric Features

Knot Diameter Slider: Adjust the cradle diameter anywhere from 22.0 mm to 34.0 mm with 0.5 mm precision.

Dual-Line Text Branding: Set Line 1 for owner name (e.g. 'TUMESH') and Line 2 for brush specification (e.g. '30mm Silvertip', 'Chubby 2', 'Omega 10049').

Built-in Vector Logos: Select from built-in emblems (Brush, Razor, Crown, Star) or import any custom vector logo via SVG ('my_logo.svg').

Flexible Logo Placement: Place emblems Left, Right, Above, or Below text, or specify exact custom (X, Y) coordinates.

Orientation Toggle: Switch between 'Flat-Side (Fast 2.5h)' and 'Upright (Standard 4.2h)' at the click of a dropdown.

## 3.2 How to Use the Customizer in OpenSCAD

Step 1: Open File: Double-click 'customize_holder.scad' in OpenSCAD.

Step 2: Enable Customizer Panel: In OpenSCAD's top menu, ensure 'Window' -> 'Customizer' is checked. The settings sidebar will appear on the right.

Step 3: Adjust Settings: Type your brush model name, pick an icon, and set your knot size.

Step 4: Render & Export: Press F6 (Render). When complete, press F7 (Export as STL) and save your new model!

## 3.3 Batch Python Generation

A helper script 'batch_generate_holders.py' is also provided. By simply running 'python batch_generate_holders.py', it will automatically render multiple customized holders defined in your preset dictionary in one single command using OpenSCAD CLI.

# 4. Slicing Optimization & 4.2° Draft Angle Correction

Standard 90° rotation in 3D slicers caused 'floating region' errors because the original model's side wall has a 4.2° draft angle (normal vector: [0.074, 0.997, 0.033]).

The OpenSCAD script incorporates the precise transformation matrix to cancel out this draft angle, seating the entire 17.32 cm² flat side flush onto the build plate at Z = 0.00 mm. This achieves rock-solid bed adhesion while reducing print height from 135.7 mm to 68.6 mm (~2.5 hours print time).

# 5. Recommended Bambu Studio Settings

| Parameter | Recommended Setting | Rationale |
| --- | --- | --- |
| Support Enable | Checked (Yes) | Required to hold overhangs. |
| Support Type | Tree (auto) or Tree Organic | Uses 70% less filament; detaches cleanly in one pull. |
| Support Style | Tree Default / Tree Slim | Lightweight branches with tiny contact points. |
| Threshold Angle | 45° - 50° | Supports only true overhangs; ignores self-supporting slopes. |
| Wall Loops | 3 to 4 perimeters | Guarantees strong, rigid brush arms and solid perimeter skin. |
| Infill Density | 20% - 25% | Optimal balance of strength, print time, and stability for wet environments. |
| Infill Pattern | Gyroid | Equal strength in all directions; prevents nozzle scraping during fast travels. |
| Recommended Material | PETG (or Matte PLA) | PETG offers superior water, soap, and temperature resistance in bathrooms. |

# 6. Project File Manifest

| File Name | Type | Description |
| --- | --- | --- |
| customize_holder.scad | OpenSCAD Script | Interactive Customizer for creating holders with custom text, logos, and knot sizes. |
| holder_template_upright.stl | STL Template | Clean, blank solid base template used by the OpenSCAD customizer. |
| batch_generate_holders.py | Python Script | Batch automated generation script for multiple shaving brushes. |
| my_logo.svg | Vector SVG | Sample vector logo badge for custom SVG import. |
| Halter_30mm_Tumesh.stl | STL Mesh | Primary 30.0 mm model (Flat-Side orientation, ~2.5h print). |
| Halter_30mm_Tumesh.step | STEP Solid | Master CAD solid file for 30.0 mm model. |
| Halter_24mm_Tumesh.stl | STL Mesh | Primary 24.0 mm model for 22mm knot + play (Flat-Side orientation). |
| Halter_24mm_Tumesh.step | STEP Solid | Master CAD solid file for 24.0 mm model. |
| Halter_28.5mm_Tumesh.stl | STL Mesh | 28.5 mm model (Flat-Side orientation). |
| Halter_28mm_Tumesh.stl | STL Mesh | Compatibility model (reloads automatically into saved .3mf project). |